University of Banja Luka

Faculty of Electrical Engineering

Department of Automatic Control

The written exam in

AUTOMATIC CONTROL SYSTEMS (P551)

17.02.2016.

- [13] Transfer function of a system in an open loop is:

Ws=1s+1

Determine and draw the response of such system if the input signal is as on Fig. 1.1.

Figure 1.1 Input signal

- [12] Transfer function of a system in an open loop is:

Ws=22s3+4s2+2s-1

Determine the stability of that system using the Nyquist criterion.

- [14] Transfer function of a system in an open loop is:

Ws=K(s+1)(s+2)(s+3)

Draw the root locus diagram of that system and, based on that, design the cascade compensator so that the compensated system reaches the stability limit for 2 times smaller gain K than the original (uncompensated) system.

- [11] For a system whose block-diagram is shown in Fig. 4.1, the following stands G1s=8s2+4s+8 and G2s=1s .

Figure 4.1 Block-diagram of a system

Design the state feedback so that the poles of a compensated system be real and have the same modules as the poles of a uncompensated system, and so that the compensated system is stable.

The tasks 1. and 2. are from the first part of the written exam, while the tasks 3. and 4. are from the second part. Maximum number of points achievable for each task is given in brackets at the beginning of that task. Minimum number of points necessary to pass a part of the written exam is 13. To qualify for the oral part of the exam, one must pass both parts of the written exam. Time available to do one part is 90 minutes, or 180 minutes for both parts.